Report 01

Exploration of Satellite Services in Brazil by Spacesail

09/09/2025

Executive Summary

This report outlines the regulatory and operational process required for Spacesail to operate in Brazil. Entry into the market depends on two different authorizations from Anatel:

1. Foreign Satellite Exploitation Rights (Landing Rights) – enables a Brazilian company to represent the foreign operator and sell satellite capacity.

2. Authorization for the Provision of Telecommunications Services – allows a Brazilian provider to offer services to the end user.

In addition to these authorizations, the process involves certification and approval of equipment, licensing of stations and compliance with applicable regulations.

1. Introduction

The exploration of satellite services in Brazil by Spacesail requires two separate authorizations from Anatel:

- Satellite Exploration Rights (Landing Rights): granted to a Brazilian company representing Spacesail China. It allows the sale of satellite capacity (infrastructure) to local providers.

- Service Authorization: necessary to offer telecommunications services to the end user, such as:

- Multimedia Communication Service (SCM),

- Personal Mobile Service (SMP),

- Global Mobile Satellite Service (SMGS).

These authorizations are complementary and indispensable. The report details the requirements and steps for each.

> Note: This document deals only with foreign satellites.

2. Right to Exploit Foreign Satellite

2.1 Corporate structure

- Creation of the Brazilian company Spacesail Brazil Holding (SBH) linked to Spacesail China.

- SBH registration  with the CNPJ.

- Appointment of SBH as the legal representative of the foreign operator.

- Appointment of SBH's legal representative  with Anatel.

2.2 Required documentation

The SBH must present:

1. Valid registration with the CNPJ.

2. Articles of incorporation of the company.

3. Declaration of not being prohibited from bidding or contracting with the Government.

4. Proof of legal and technical qualification, economic and financial capacity and tax regularity.

5. Simplified technical design of the satellite communication system.

6. Declaration of compliance with the applicable regulations.

7. Document issued by the competent body in the country of origin (filing) with a sworn translation.

8. Agreements or proof of frequency coordination efforts.

9. Colocation agreements, if applicable.

10. Indication of the company responsible for paying the Fistel fees.

11. Simplified technical information of the satellite system:

- System name,

- Orbital position,

- Number of associated satellites,

- Satellite networks in the ITU (filings),

- Launch and start of operation dates in Brazil,

- Orbital control accuracy,

- Gateways planned in Brazil,

- Coverage area,

- Frequency bands (start, end, rise/fall, bias),

- TT&C stations (telemetry, tracking and control),

- Orbital parameters (planes, inclination, altitudes, satellites per plane).

2.3 Frequency coordination

- SBH should start negotiations with other operators.

- Records of efforts (minutes, reports, videos, etc.) must be presented.

- Anatel's authorization will be:

- Primary, when coordination is completed.

- Secondary, when there is still no agreement, without protection against interference.

2.4 Costs

- Fee for the right to exploit: R$ 102,677.00.

3. Authorization to Provide Services to the End User

3.1 Corporate structure

- Creation of the company Spacesail Brazil Serviços (SBS), registered with the CNPJ.

- SBS will be responsible for offering services to subscribers.

3.2 Possible services

- SCM (Multimedia Communication Service): fixed, flexible and comprehensive broadband.

- SMGS (Global Mobile Satellite Service): direct communication between terminals and satellites.

- SMP (Personal Mobile Service): mobile communications service, including future applications of *Direct to Device (D2D).

3.3 Required documentation

SBS must present:

1. Valid registration with the CNPJ.

2. Consolidated and registered articles of incorporation, including shareholding composition and election of managers (when applicable).

3. Declaration of incorporation under Brazilian laws and headquarters in the country.

4. Declaration of suitability (not prevented from contracting, not declared unsuitable).

5. Declaration of technical, economic qualification and tax regularity.

6. Declaration of not holding other collective service authorizations.

3.4 Costs

- SCM Authorization: R$ 400.00.

> Note: Companies with less than 5% of market share in the national market can be classified as Small Providers (PPP) and receive simplified regulatory treatment.

4. Equipment Certification and Approval

- All network and user equipment must be certified by Anatel.

- Process conducted by Designated Certification Bodies (OCDs). CPQD will provide a list with all documents needed and conduct the process for certification.

- Normative basis: Regulation for Conformity Assessment and Approval of Telecommunications Products (RAC).

5. Station Licensing

5.1 Definitions

- Telecommunications Station: set of telecommunications equipment and installations.

- Earth Station: located on the Earth's surface, it communicates with satellites.

- Gateway: access station that connects satellites to terrestrial networks.

5.2 Procedure

- Registration of stations in Anatel's BDTA.

- Licensing request via electronic system.

- The process follows the guidelines of the General Licensing Regulation (RGL).

- Licensing can be individual or block.

5.3 Costs

- Installation Inspection Fee (TFI): R$ 26,816.00 per space station.

- Operating Inspection Fee (TFF): 50% of the TFI, annual.

5.4 Observations

- For non-geostationary systems, licensing is done per system, simplifying the process.

- Licensing can cover more than one service, with only the highest fee applying.

-  Commercial operation can only start after the licensing of all stations.

6. Applicable Regulations

| Norm | Object |

|----------|----------|

| Law No. 9,472/1997 | General Telecommunications Law (LGT) – right to exploit satellite |

| Resolution No. 748/2021 | General Regulation for the Exploitation of Satellites |

| Act No. 9,523/2021 | Technical and Operational Requirements for Satellite Communication Systems |

| Act No. 9,426/2021 | Technical parameters for coexistence between ground and satellite stations |

| Act No. 9,526/2021 | Requirements for obtaining the right to exploit satellites |

| Law No. 5,070/1966 | Fistel Law – defines TFI and TFF |

| Resolution No. 715/2019 | Equipment Certification and Approval Regulation |

| Resolution No. 719/2020 | General Licensing Regulation (RGL) – defines rules for station licensing |

7. Conclusion

To operate in Brazil, Spacesail must comply with a structured regulatory process that includes:

1. Obtaining the Right to Exploit a Foreign Satellite (SBH).

2. Obtaining Service Authorizations (SBS).

3. Certification and approval of equipment.

4. Licensing of ground and space stations, in accordance with the RGL.

Compliance with these steps ensures legal and technical compliance for entry into the Brazilian satellite telecommunications market.